Payload Electronics — Component Research

Sensor picks, system diagram, MCU choice, storage plan, risks, and fall next steps

# Sensors

## Pressure & temperature

Same chip measures both.

| Spec | BMP390 |
| --- | --- |
| Accuracy | ±3 Pa (≈ 0.25 m altitude) |
| Speed | up to ≈200 Hz |
| Interface & power | I2C/SPI, <650 µA |
| Cost | ≈ $10 |

## Vibration

Measures shock and vibration through flight, including ignition and parachute deployment.

| Spec | ADXL375 |
| --- | --- |
| Range | ±200 g |
| Bandwidth | ≈ 1 kHz |
| Interface & power | I2C/SPI, ≈145 µA |
| Cost | ≈ $15 |

Pick: ADXL375 — ±200 g range, won't clip on shock.

## Humidity

Adds relative humidity to the pressure/temperature reading.

| Spec | SHT40 |
| --- | --- |
| Accuracy | ±1.8% RH, ±0.2 °C |
| Also measures | Humidity & temperature only |
| Interface & power | I2C, low power |
| Cost | ≈ $6–$8 |

# System Diagram

Power flows from the regulator to both sensors and the microcontroller. Sensors report over I2C/SPI to the Teensy, which timestamps and logs each sample to the microSD card. USB handles programming; the avionics link is optional.

# Microcontroller: Teensy 4.1

| Spec | Teensy 4.1 |
| --- | --- |
| Speed | 600 MHz, single core |
| Memory | 1 MB RAM / 8 MB flash |
| SD card | Built-in, fast slot |
| Radio | None — quieter near GPS/altimeter |
| Timing | Nothing competes for CPU time |
| Price | ≈ $31.50 |

Pick: Teensy 4.1 — cleaner sampling loop, built-in SD slot, no radio nearby. ESP32 is the pick for wireless telemetry or a tight budget.

# Storage Plan

| Channel | Rate | Data per flight |
| --- | --- | --- |
| Pressure & temp | 100 Hz | ≈ 0.6 MB |
| Vibration | 800 Hz | ≈ 4.8 MB |
| Total (packed binary) | — | ≈ 5.4 MB |

As plain-text CSV instead of packed binary: ≈ 30–45 MB per flight. An 8–32 GB microSD card covers a full season many times over. The Teensy timestamps every sample from one shared clock, so pressure and vibration data stay aligned.

# Major Risks

- Sensor clipping — not enough range cuts off shock data.

- Power sag at ignition — give the regulator margin to avoid a reset.

- Slow SD writes — buffer in RAM so a slow write doesn't drop a sample.

- Loose wiring — use locking connectors and strain relief.

- Shared-bus lockup — split sensors across separate I2C/SPI buses.

- Corrupted log file — flush/close the file periodically.

- Radio noise — keep sensor wiring short and away from trackers or altimeters.

# Fall Next Steps

- Breadboard the MS5611 + ADXL375 on the Teensy 4.1 and confirm logging works at full speed.

- Measure real power draw, size the battery and regulator, and test recovery from a brief power dip.

- Shake-test the prototype (or fly it non-critically) to confirm mounting and sample rates actually capture what we need.

# Sources

Datasheets and vendor pages: TE Connectivity/Amsys MS5611-01BA03; Bosch Sensortec BMP390 and BME280; Analog Devices ADXL375 and ADXL377; TDK InvenSense ICM-42688-P; Sensirion SHT4x; u-blox NEO-M9N and NEO-M8N; PJRC/SparkFun Teensy 4.1; Espressif ESP32-DevKitC-32E.